Command line parameters documentation for:                                 94-05-14

LOADLIN 1.4 ((C) 1994 lermen@elserv.ffm.fgan.de)

and

Linux version: 0.99.14, 0.99.15, 1.0

This is a list of command line parameters recognized and interpreted by:

========================================================================

1. LOADLIN

ramdisk=size        (size in Kbytes)

vga=mode

2. The Linux kernel

mem=number       (for > 0.99.15,  number = end of physical memory in bytes)

root=device

ro

rw

no387

3. init

single

auto

ro,rw,no387   (forwarded from the kernel)

4. Some drivers

ether=irq,base_addr,mem_start,mem_end,dev_name

reserve=port1,num1,...,port5,num5

hd=cyl,head,sect

bmouse=irq

st0x=base_address,irq                    # SEAGATE controller ST01/ST02

tmc8xx=base_address,irq                  # Future Domain TMC-885,TMC-950

t128=address,irq                         # Trantor T128/T128F/T228

ncr5380=port,irq,dma                     # Generic NCR5380 driver

aha152x=portbase,irq,scsiid,reconnect    # Adaptec AHA-152x driver

xd=type,irq,iobase,dma

mcd=port,irq

sound=0x0TTPPPID, ..

sbpcd=sbport,SoundBlaster

sbpcd=LMport,LaserMate

5. Environment

All parameters of format  name=xxxx  which are not recogized by

the kernel or a driver are put onto the environment.

LOADLIN itself always puts  BOOT_IMAGE=imagefile  onto the environ-

ment.  The user may put additional values onto the environment in

order to allow  /etc/rc  to behave differently for a given condition.

==============================================================================

Following are detailed descriptions:

Parameters recognized by LOADLIN:

=================================

root=device

-----------

device     may be a symbolic device name such as "/dev/hda2"

or the numeric device number (hex)

(highbyte=major, lowbyte=minor)

recognized symbolic devices (with their partitions) are:

"hda", "hdb", "sda", "sdb", "sdc", "sdd", "sde",

"fd", "xda", "xdb"

There is a problem:

Linus interprets the number following "/dev/hda" as a HEX number,

The device names in  /dev  of the Slackware distribution, however,

are numbered /dev/hda1 .. hda9 .. hda10 .. hda16,

instead of   /dev/hda1 .. hda9 .. hdaa ..  hdaf  as Linus would have done.

So, starting with version 1.3 of LOADLIN, I interpret the root device

myself and pass the device numerically in the boot sector.

My naming convention is a compromise which I hope will result in

fewer irritations:

If the number following "/dev/hda, hdb, sda ..."

starts with 0, 0x or a..f, it is interpreted as a HEX number.

If it starts with 1..9 it is interpreted as a decimal number.

If none is appropriate, the string is passed to the command line.

(some later kernel versions may have other device names).

Examples:

root=302

root=0x201

root=/dev/hda2

root=/dev/sdaa   same as  root=/dev/sda10  or  /dev/sda0xa

ramdisk=size

------------

size      number of Kbytes for the RAMDISK device.

The image for the ramdisk is loaded by the

kernel at startup from a floppy drive, if

root=/dev/fdx  is also given.

After loading the image, the kernel then makes

the ramdisk the root device (NOT the floppy).

In addition to this, LOADLIN prompts for inserting

the ramdisk image into the floppy drive if it no

longer needs the drive (i.e. after loading the kernel).

vga=mode

--------

mode      -3  the user is asked to specify the VGA mode

ask  equal to -3

-2  80x50 extended VGA mode

extended  equal to -2

-1  80x25 normal mode

normal  equal to -1

0  as if "0" was entered at the boot prompt

.

.

n  as if the number "n" was entered at the boot prompt

ro

--

"ro" forces the root device to be mounted read-only (Not for UMSDOS!)

rw

--

"rw" forces the root device to be mounted read/write.

------------------------------------------------------------------------------

Parameters recognized only by the kernel:

=========================================

mem=number

----------

number     End of physical RAM, this is recognized by kernels

of 0.99.15 and higher and is useful if the exact

installed RAM size cannot be determined automatically.

root=device

-----------

device     May be a symbolic device name such as "/dev/hda2"

or the numeric device number (highbyte=major, lowbyte=minor)

Recognized symbolic devices (with their partitions) are:

"hda", "hdb", "sda", "sdb", "sdc", "sdd", "sde",

"fd", "xda", "xdb"

example:

root=302

root=/dev/hda2

!!      NOTE:   Older kernels ( below Linux 1.0.5 ) have a little bug in

!!              (init/main.c), but it is of little concern:

!!               The last part of the symbolic device name ("hda2") is

!!               passed as option parameter to  init, so init will

!!               say "illegal option", but continue without errors.

!!

!!              Of greater concern is the too-small static buffer

!!              "command_line" (80 bytes, in init/main.c).   It is

!!              preferable to set it to 512 and recompile the kernel.

!!

!!              This bug is fixed with Linux 1.0.5, the "command_line"

!!              is now 256 by default and (if needed ) can easily be changed

!!              with   #define COMMAND_LINE_SIZE xxxx  in init/main.c  (to a

!!              maximum size of 2047).

no387

-----

"no387" disables use of the hardware math coprocessor.

------------------------------------------------------------------------------

Option Parameters recognized by "init":

=======================================

single

------

Enter "single user mode" on boot

auto

----

ro,rw,no387   (forwarded from kernel).   (see !!comment!! above)

-----------

------------------------------------------------------------------------------

Configuration of ethernet cards:

================================

ether=irq,base_addr,mem_start,mem_end,dev_name

---------------------------------------------

irq        number, IRQ-number (3..15)

base_addr  number, IO-base (0x280,...)

mem_start  number, shared mem start

mem_end    number, shared mem end

dev_name   string,

eth0..3     all configured ethernetcards

dl0         D-Link DE600 pocket adaptor

atp0        AT-LAN-TEC (RealTek) pocket adaptor

plip0..2    parallel IP (PLIP)

sl0..3      serial IP (SLIP)

example:

ether=10,0x280,0xc8000,0xcbfff,eth0

related files: net/inet/eth.c, drivers/net/Space.c

------------------------------------------------------------------------------

Reserving I/O port space:

=========================

reserve=port1,num1,...,port5,num5

---------------------------------

portn     first IOport-address in range

numn      number of addresses in this range to reserve

(there will be bits set to "1" in "ioport_register")

related files:  kernel/ioport.c

------------------------------------------------------------------------------

Setting hard disk parameters for the root device:

=================================================

hd=cyl,head,sect

---------------

related files: drivers/block/hd.c

config:        CONFIG_BLK_DEV_HD

------------------------------------------------------------------------------

Setting the IRQ for the bus-mouse:

==================================

bmouse=irq

----------

related files: drivers/char/busmouse.c

config:        CONFIG_BUSMOUSE

------------------------------------------------------------------------------

Parameters for SCSI controllers:

================================

st0x=base_address,irq                     SEAGATE controller ST01/ST02

tmc8xx=base_address,irq                   Future Domain TMC-885,TMC-950

t128=address,irq                          Trantor T128/T128F/T228

ncr5380=port,irq,dma                      Generic NCR5380 driver

aha152x=portbase,irq,scsiid,reconnect     Adaptec AHA-152x driver

related files: drivers/scsi/seagate.c, t128.c, g_NCR5380.c, aha152x.c

config:        CONFIG_SCSI_SEAGATE, CONFIG_SCSI_T128,

CONFIG_SCSI_GENERIC_NCR5380, CONFIG_SCSI_AHA152X

------------------------------------------------------------------------------

Parameter for XT hard disk controller (DTC 5150X):

==================================================

xd=type,irq,iobase,dma

----------------------

related files: drivers/block/xd.c

config:        CONFIG_BLK_DEV_XD

------------------------------------------------------------------------------

Parameter for Mitsumi CDROM driver:

===================================

mcd=port,irq

------------

related files: drivers/block/mcd.c

config:        CONFIG_MCD

------------------------------------------------------------------------------

Parameter for sound cards:

==========================

sound=0x0TTPPPID, ..

----------------

all values are enbedded in one longint, the above format

shows a hex number, each place is a hex-digit

TT         Supported card type.

This is the index into the list of configured cards

If all cards are configured (which normally is not the case)

the following order is kept:

Roland MPU-401

Gravis Ultrasound

ProAudio Spectrum

SoundBlaster

SoundBlaster 16

SB16 MPU-401 (SB16-MIDI port)

AdLib

PPP        Port Address

I          IRQ

D          DMA Channel

related files: drivers/sound/dev_table.h,dev_table.c

config:        CONFIG_SOUND

------------------------------------------------------------------------------

Parameter for IDE-style CD-ROM device driver (Kotobuki/Matsushita/Panasonic):

=============================================================================

sbpcd=sbport,SoundBlaster

------------------------

sbpcd=LMport,LaserMate

----------------------

sbport  soundblaster base port +0x10 (i.e 0x230 = 0x220+0x10)

LMport  MPU-type port at 0x300 or equivalent

related files: drivers/block/sbpcd.c

config:        CONFIG_SBPCD

------------------------------------------------------------------------------